Electronic Devices and Circuits

EE-1004

Lab Project

Fall 2025

Construction a Common Source Amplifier for given specifications

Submitted by:

Muhammad saad khan 24i-6550.

Abdul hadi         24i-6538.

National University of Computer and Emerging Sciences, Islamabad

Final project report :

Introduction:

In this project , a single stage common source amplifier is used with a MOSFET.

A 3 inch by 3 inch PCB is used. The pcb is designed then etched, drilled and after it placed the components on it. The board has MOSFET , load and source   resistors according to the calculations ,coupling and bypass capacitors, connectors and the connectors for the input output signal  . most parts are being placed compactly so that the signal flows very clearly without any kind of noise .

After soldering , we had the working amplifier circuit with 24mV peak to peak sine wave with a frequency of 50 hz. Then 18V supply is provided and then to measure the different voltages , probes are placed at various points on the circuit. Final circuit looks like a small , neatly wired circuit that is measured in the lab.

Calculations :

Parts list:

1 X BS170 mosfet .

1 X 100 KOHMS fixed resistor.

1 X 100 KOHMS variable resistor (potentiometer).

RD: 15 K ohms

RS: 15KOhms.

1X input coupling capacitor 33nf.

1x output coupling capacitor 100nf.

1x source bypass capacitor 1microfarad,

18 v dc supply .

PCB materials:

Single sided 3by3 inch pcb.

Permanent marker.

Sand paper.

Ferric chloride as alkaline solution for etching.

Cotton and nail polish remover.

Tools:

Soldering iron and solder.

Pcb drill.

Results:

Simulation:

Vg= 9 v

Vd= 12.9 v

Vs=6.32v

Vgs=1.99v

Vds=5.7v

Id=0.64ma

Av =Vout /Vin=272/8=34v/v.

In practical circuit:

Vg=8.12v

Vd=12.67v

Vs=6.03v

Vds=5.2v

Vgs=2.4v

Id=.59ma

Observation:

Simulation results are very close to the practical ones and the values  and calculated values.

Stable output gain is obtained and 180 degree phase shift is observed too.

DISCUSSION:

The circuit behaved in the same way as expected as the common source ampklifeir should work .real world results lined up in a good way for what we calculated and simulated.once bias was adjusted , MOSFET settled closed to the calculated values output showed a clean , amplified signal with great 180 degree phase shift.

The gain came out as expected according to the calculations greater than 22 . so our gain is 34 V/V. So, input output impedence lies ion the expected range. Fine tuning further cleared the signal as shown in the pic below :

Pics:[

Conclusion:

A few practicval issues including the loose coonections to the pcb were faced at first but after it they were resolved.oscilloscope probes had loose pins , which at first introduced noise and output was disturbed . it was resolved after using secured probes and switching to better pair of probes.

So bacsically , expected values were observed and a great gain there too .

Attachments: